Electric Vehicle Assembly Logistics Readiness Case Study

BOM, Supplier, Inventory, Kitting, Shortage, Risk, and Tableau Dashboard Analysis

Prepared by Shiven Parikh

Model Type: Mock EV assembly logistics model for a 25-vehicle production batch

Workbook analyzed: Electric Vehicle Assembly Logistics Case Study - BOM.xlsx

Dashboard tool: Tableau

Figure 1. Tableau dashboard created from the EV assembly logistics workbook.

# Table of Contents

1. Executive Summary

2. Project Background and Business Problem

3. Scope, Assumptions, and Data Model

4. Workbook Architecture

5. Methodology by Sheet

6. Tableau Dashboard Design

7. Key Findings

8. Operational Interpretation

9. Recommendations

10. Limitations and Future Improvements

Appendix A. Shortage Tracker Summary

Appendix B. Risk Scoring Model Summary

Appendix C. Portfolio Positioning

# 1. Executive Summary

This case study models the material readiness process for a mock electric vehicle assembly operation. The project connects a simplified bill of materials, supplier assignments, inventory availability, kitting readiness, active shortages, supplier and production risk, and a Tableau operations dashboard. The purpose is to show how a manufacturing supply chain team could evaluate whether a 25-vehicle EV production batch is ready to move through assembly.

The workbook contains six linked sheets: a Simplified BOM, Supplier Master, Inventory Master, Kitting Readiness Tracker, Shortage Tracker, and Risk Scoring Model. The Tableau dashboard converts those sheets into an executive readiness view showing kit status, shortage concentration, top production risks, risk category distribution, and shortage recovery actions.

The model identifies 102 tracked components and internal subassemblies across the battery pack, electric drive unit, power electronics, wiring, chassis/body, thermal system, and suspension/brake/wheel systems. Out of the 102 tracked kitting rows, 85 are complete and 17 are on hold because required material could not be fully picked. All 17 shortages are treated as line-stopper shortages because they affect required assembly steps. The dashboard shows that the Battery Pack System has the largest shortage quantity, with 526 missing units out of 784 total missing units across the batch.

This is not a live production system or an official EV manufacturer BOM. It is a portfolio case study built to demonstrate practical supply chain thinking: BOM structure, supplier mapping, inventory logic, kitting logic, shortage escalation, risk scoring, and dashboard communication.

## Executive KPI Summary

| Metric | Value |
| --- | --- |
| Tracked kitting components | 102 |
| Complete kitting rows | 85 |
| Kitting rows on hold | 17 |
| Total kit IDs | 9 |
| Kit IDs with at least one hold item | 8 |
| Active shortage rows | 17 |
| Line-stopper shortages | 17 |
| Total missing quantity | 784 |
| Average risk score | 3.09 |
| Critical risk components | 8 |
| High risk components | 15 |

# 2. Project Background and Business Problem

Electric vehicle production is a complex assembly environment. A finished EV depends on thousands of interactions between engineered components, purchased parts, internal subassemblies, supplier deliveries, warehouse operations, line-side staging, and production schedules. In this type of environment, a single missing part can delay a full assembly stage even when most of the bill of materials is available.

The business problem modeled in this project is simple: the plant needs to know whether the material required for a 25-vehicle EV build can be picked, kitted, and released to the assembly line. If the answer is no, the team needs to know what is missing, which assembly area is affected, who owns the recovery action, and which shortages carry the highest production risk.

The case study moves beyond a static BOM by modeling the operational flow of material readiness. It does this by connecting material requirements, available inventory, kitting results, shortage escalation, and risk scoring into one supply chain readiness dashboard.

## Core business questions

What components are required to support a 25-vehicle EV production batch?

Which items are purchased externally, internally assembled, or treated as stocked material?

Which inventory rows have enough available material to support the batch?

Which kits can be released to production and which kits are on hold?

Which missing components are line stoppers?

Which owners are responsible for recovery actions?

Which components carry the highest combined supplier, inventory, substitution, shortage, and line-stopper risk?

# 3. Scope, Assumptions, and Data Model

## Scope

The project focuses on material readiness for a mock electric vehicle assembly process. The model includes major EV systems such as the battery pack, electric drive unit, power electronics and charging system, wiring and control system, chassis and body structure, thermal management system, and suspension/brakes/wheels. The goal is not to recreate a confidential OEM BOM. The goal is to build a realistic learning model that shows how supply chain logic connects to complex assembly operations.

## Main assumptions

The production batch size is 25 electric vehicles.

The workbook uses a simplified EV BOM rather than a full OEM-grade engineering BOM.

The first eight inventory rows are final/summary assembly rows and are not treated as stock-tracked components.

The kitting tracker includes 102 stock-tracked components, internal subassemblies, consumables, or hardware rows.

A kitting row is marked Complete when the picked quantity equals the required quantity.

A kitting row is marked Hold when missing quantity is greater than zero.

The shortage tracker includes only rows where Missing Qty is greater than zero.

The risk scoring model uses a 1-5 scoring scale and a weighted score, not live supplier performance data.

The Tableau dashboard is intended as an operations review dashboard, not a financial valuation model.

## Data flow

The workbook follows a supply chain logic flow:

| Step | Model Layer | Purpose |
| --- | --- | --- |
| 1 | BOM Master | Defines the EV component structure, categories, quantities, and assembly grouping. |
| 2 | Supplier Master | Assigns suppliers, regions, source confidence, risk factors, and sourcing strategies. |
| 3 | Inventory Master | Calculates required quantity, available inventory, safety stock, reorder point, shortage quantity, and inventory status. |
| 4 | Kitting Readiness Tracker | Determines whether required material can be picked into production kits. |
| 5 | Shortage Tracker | Filters active shortages and assigns severity, root cause, recovery action, owner, and production impact. |
| 6 | Risk Scoring Model | Scores components using lead time, inventory, supplier, substitution, line-stopper, and shortage risk. |
| 7 | Tableau Dashboard | Visualizes readiness, shortages, risk concentration, and recovery actions. |

# 4. Workbook Architecture

| Workbook Sheet | Approx. Range | Role in the Case Study |
| --- | --- | --- |
| Simplified BOM | A1:P111 | Defines product structure, component/material, quantity per vehicle, category, cost estimates, assembly stage, and criticality. |
| Supplier Master | A1:T101 | Maps buy items to likely supplier names, source confidence, supplier region/country, backup supplier, MOQ, cost, supplier risk, and sourcing strategy. |
| Inventory Master | A1:R111 | Tracks item type, required quantity, inventory, allocation, available inventory, safety stock, reorder point, shortage quantity, inventory status, and lot/serial tracking. |
| Kitting Readiness Tracker | A1:N103 | Converts inventory availability into kit-level production readiness for 102 tracked rows. |
| Shortage Tracker | A1:S22 | Creates an action tracker for the 17 active shortage rows. |
| Risk Scoring Model | A1:M103 | Calculates component risk using weighted scoring across six risk dimensions. |

The workbook is designed as a layered model. Each sheet has a specific operational purpose. The BOM defines what is needed. The supplier master explains where components come from. The inventory master determines what is physically available. The kitting tracker determines whether assembly kits can be released. The shortage tracker explains what must be fixed. The risk scoring model prioritizes which issues require management attention.

# 5. Methodology by Sheet

## 5.1 Simplified BOM

The Simplified BOM is the foundation of the project. It organizes the vehicle into major assemblies and component rows. The major assemblies include the Battery Pack System, Electric Drive Unit, Power Electronics and Charging System, Wiring and Control System, Chassis and Body Structure, Thermal Management System, and Suspension, Brakes, and Wheels.

The BOM is not only a cost list. In a manufacturing logistics context, the BOM is the demand signal that drives material planning. Qty per Vehicle determines how many units of each part are needed for the planned production batch.

### BOM logic used

Major Assembly groups components into EV systems.

Qty per Vehicle creates the demand basis for a production batch.

Make/Buy and item type separate internally assembled items from purchased parts.

Assembly stage connects components to production flow.

Criticality supports prioritization when shortages occur.

## 5.2 Supplier Master

The Supplier Master converts the component list into a sourcing view. Each component is assigned a supplier, source confidence level, supplier region, country, backup supplier, lead time estimate, minimum order quantity, risk factors, and sourcing strategy. This creates the supplier-side context needed to understand which components are exposed to delivery risk, capacity risk, sourcing risk, or substitution difficulty.

Supplier Master rows are intentionally separated from the BOM. This is better modeling practice because a BOM should describe product structure, while a supplier master should describe sourcing relationships and vendor risk.

### Supplier Master fields

| Field Group | Examples | Purpose |
| --- | --- | --- |
| Supplier identity | Supplier ID, Supplier Name, Country, Region | Identifies source of supply and supplier geography. |
| Sourcing confidence | Source Confidence, Primary/Backup, Backup Supplier | Shows how reliable the sourcing assumption is. |
| Commercial and supply factors | Lead Time, MOQ, Unit Cost | Supports planning and procurement decisions. |
| Risk factors | Reliability, Quality, Cost, Geopolitical, Substitution Difficulty | Supports supplier risk scoring. |
| Strategy | Single source, dual source, validation required | Turns supplier information into an action recommendation. |

## 5.3 Inventory Master

The Inventory Master tests whether enough material exists to support the 25-vehicle build. It tracks required quantity, current inventory, allocated inventory, available inventory, safety stock, reorder point, shortage quantity, inventory status, and lot/serial tracking requirements. This sheet is where the project shifts from product structure to operational readiness.

### Inventory formulas

| Inventory Field | Logic | Meaning |
| --- | --- | --- |
| Required Qty | Qty per Vehicle x Planned Production Qty | Calculates demand for the 25-vehicle batch. |
| Available Inventory | Current Inventory - Allocated Inventory | Shows stock that can actually be used for this build. |
| Safety Stock | 20% of Required Qty | Creates a simple buffer for production protection. |
| Reorder Point | Required Qty + Safety Stock | Flags when replenishment should be triggered. |
| Shortage Qty | MAX(0, Required Qty - Available Inventory) | Calculates the quantity that cannot be fulfilled. |
| Inventory Status | OK / Reorder Needed / Shortage | Summarizes whether material is ready, low, or insufficient. |

### Inventory status result

| Inventory Status | Count |
| --- | --- |
| OK | 70 |
| Reorder Needed | 15 |
| Shortage | 17 |

The inventory model produced 70 OK rows, 15 Reorder Needed rows, and 17 Shortage rows. The 17 shortage rows become the input for the shortage tracker and several Tableau dashboard visuals.

## 5.4 Kitting Readiness Tracker

The Kitting Readiness Tracker converts inventory availability into production kit readiness. Kitting is the operational step where warehouse teams pick and group components for a specific assembly stage. A kit is not ready just because a component exists somewhere in inventory. It is ready only if the required quantity can be picked and staged for the production batch.

### Kitting formulas

| Kitting Field | Logic | Meaning |
| --- | --- | --- |
| Picked Qty | MIN(Required Qty, Available Inventory) | Picks the maximum possible quantity without exceeding demand. |
| Missing Qty | MAX(0, Required Qty - Picked Qty) | Shows the quantity still missing from the kit. |
| Kit Status | Complete if Missing Qty = 0; Hold if Missing Qty > 0 | Determines whether the kit can be released to production. |
| Line Stopper? | Yes/No based on production criticality | Identifies whether a shortage can block assembly. |
| Responsible Owner | Warehouse, buyer, supplier planner, quality, production planner, or maintenance planner | Assigns accountability for completion or recovery. |

### Kitting readiness result

| Kitting Result | Count | Percent of Components |
| --- | --- | --- |
| Complete | 85 | 83.3% |
| Hold | 17 | 16.7% |
| Total | 102 | 100.0% |

The model found 85 complete kitting rows and 17 hold rows. There are 9 kit IDs, and 8 of them contain at least one hold item. This does not mean every kit is completely unusable, but it does mean those kits require intervention before full release.

## 5.5 Shortage Tracker

The Shortage Tracker is an exception-management sheet. It filters the model down to the rows where Missing Qty is greater than zero. Instead of reviewing all 102 kit rows, the operations team can focus only on the 17 rows that require action. This is realistic because supply chain control teams usually manage by exception: what is late, missing, blocked, or at risk.

### Shortage tracker fields

| Field | Purpose |
| --- | --- |
| Shortage ID | Creates a unique issue number for tracking. |
| Kit ID and Assembly Stage | Shows where the shortage affects the production process. |
| Component / Material | Identifies the missing item. |
| Required, Available, Picked, Missing Qty | Quantifies the gap. |
| Line Stopper? | Shows whether the shortage can block production. |
| Shortage Severity | Prioritizes issue seriousness. |
| Responsible Owner | Assigns accountability. |
| Root Cause | Explains why the shortage exists. |
| Recovery Action | Defines the immediate response. |
| Expected Recovery Date | Sets a target for issue closure. |
| Escalation Status | Shows whether the issue has been escalated. |
| Production Impact | Summarizes the assembly consequence. |

## 5.6 Risk Scoring Model

The Risk Scoring Model expands the analysis beyond current shortages. It scores all 102 tracked components, even components that are currently complete. This matters because a component can be risky even before it becomes a shortage. For example, a critical EV electronics part may have no current shortage but still carry high substitution difficulty and supplier exposure.

### Risk scoring formula

The model uses a weighted score across six risk factors:

| Risk Factor | Weight | Meaning |
| --- | --- | --- |
| Lead Time Risk | 15% | Captures exposure from long or uncertain replenishment timing. |
| Inventory Risk | 25% | Captures current inventory exposure and shortage status. |
| Supplier Risk | 20% | Captures supplier reliability, quality, cost, and geopolitical exposure. |
| Substitution Difficulty | 15% | Captures how hard it would be to switch suppliers or replace the part. |
| Line Stopper Risk | 15% | Captures whether missing the part blocks assembly. |
| Shortage Risk | 10% | Captures current missing quantity exposure. |

Formula: Total Risk Score = (Lead Time Risk x 0.15) + (Inventory Risk x 0.25) + (Supplier Risk x 0.20) + (Substitution Difficulty x 0.15) + (Line Stopper Risk x 0.15) + (Shortage Risk x 0.10).

### Risk category result

| Risk Category | Component Count |
| --- | --- |
| Critical | 8 |
| High | 15 |
| Medium | 72 |
| Low | 7 |

The average total risk score across the 102 components is 3.09. The final risk distribution contains 8 Critical, 15 High, 72 Medium, and 7 Low risk components.

# 6. Tableau Dashboard Design

The Tableau dashboard converts the workbook into a visual production readiness view. The dashboard is designed for an operations review meeting where planners, buyers, warehouse teams, and production leaders need to quickly see what is ready, what is blocked, where shortages are concentrated, and who owns the recovery actions.

Figure 2. EV Assembly Logistics Readiness Dashboard.

## Dashboard visuals

| Dashboard Sheet | What it Shows | Why it Matters |
| --- | --- | --- |
| Kit Readiness by Assembly Kit | Shows complete vs hold rows by kit ID. | Identifies which production kits are blocked. |
| Top 10 Highest-Risk Components | Ranks components by total risk score. | Directs management attention to the highest risk items. |
| Shortage Quantity by Major Assembly | Aggregates missing quantity by EV system. | Shows where material gaps are concentrated. |
| Risk Category Breakdown | Counts components by Critical, High, Medium, and Low risk. | Summarizes total risk exposure. |
| Active Shortage Recovery Plan | Lists shortage ID, component, missing qty, severity, owner, recovery date, and escalation status. | Turns the dashboard into an action tracker. |

# 7. Key Findings

## 7.1 Kitting readiness is mostly complete, but several kits are blocked

Out of 102 tracked components, 85 are complete and 17 are on hold. This means 83.3% of the component rows can be picked successfully, while 16.7% cannot be fully picked for the batch. The most important operational point is that a small number of missing rows can still block major assembly progress.

The kit-level view is valuable because it shows how shortages are distributed across assembly kits rather than only listing individual parts. KIT-BAT-001, KIT-BAT-002, and KIT-DRV-002 carry the most visible hold pressure in the dashboard.

## 7.2 Battery Pack System has the largest shortage quantity

| Major Assembly | Total Missing Quantity | Share of Total Missing Qty |
| --- | --- | --- |
| Battery Pack System | 526 | 67.1% |
| Thermal Management System | 120 | 15.3% |
| Electric Drive Unit | 93 | 11.9% |
| Suspension, Brakes, and Wheels | 38 | 4.8% |
| Chassis and Body Structure | 6 | 0.8% |
| Power Electronics and Charging System | 1 | 0.1% |

The Battery Pack System accounts for 526 missing units, or 67.1% of total missing quantity. This is the largest shortage concentration in the model. That makes sense for an EV assembly project because battery pack components include high-volume and high-criticality material such as sensors, cooling hardware, busbars, insulation sheets, and sealing components.

## 7.3 Electric Drive Unit shortages are smaller in quantity but high in severity

The Electric Drive Unit has 93 missing units, which is lower than the Battery Pack System and Thermal Management System. However, several of the electric drive shortages are severe because they affect motor assembly, sensors, bearings, copper windings, gearbox bearings, and lubrication. This shows why production risk cannot be judged only by missing quantity. A shortage of one critical component can be more serious than a shortage of many low-risk items.

## 7.4 Shortage ownership is concentrated in supplier planning

| Responsible Owner | Shortage Count | Share of Shortages |
| --- | --- | --- |
| Buyer / Supplier Planner | 12 | 70.6% |
| Warehouse / Maintenance Planner | 2 | 11.8% |
| Warehouse / Supplier Planner | 1 | 5.9% |
| Production Planner | 1 | 5.9% |
| Quality + Supplier Planner | 1 | 5.9% |

Buyer / Supplier Planner owns 12 of the 17 shortages. This tells the operations team that the largest part of the recovery work is supplier-facing rather than purely warehouse-facing. The dashboard therefore supports a supplier escalation meeting as well as an internal kitting review.

## 7.5 Most shortages are already escalated

| Escalation Status | Count |
| --- | --- |
| Escalated | 13 |
| Open | 4 |

The Shortage Tracker shows 13 escalated shortages and 4 open shortages. This creates a clear split between issues that already require management attention and issues that are still being managed through normal follow-up.

## 7.6 Risk distribution is broad, not limited to shortages

The risk scoring model shows that most components are medium risk, but a meaningful number are high or critical. This is useful because a part may not be short today but may still need monitoring because of long lead time, high substitution difficulty, supplier exposure, or line-stopper importance.

| Risk Category | Count | Interpretation |
| --- | --- | --- |
| Critical | 8 | Immediate management attention if tied to shortage, launch, or single-source risk. |
| High | 15 | Requires active monitoring and a recovery or mitigation plan. |
| Medium | 72 | Should be tracked in weekly planning reviews. |
| Low | 7 | No immediate action required beyond standard controls. |

# 8. Operational Interpretation

The main operational story is that the production batch is partially ready. Most components can be picked, but several line-stopper shortages prevent full kit release. Battery pack shortages create the largest missing quantity exposure. Electric drive unit shortages create high severity because they affect motor and gearbox completion. The shortage recovery plan shows that most corrective actions belong to supplier planning, with some issues assigned to warehouse/maintenance, production planning, and quality.

## Readiness interpretation by function

| Function | Interpretation |
| --- | --- |
| Warehouse / Kitting | Most rows can be picked, but 17 rows cannot be fully completed. Kits with holds should not be released without an approved recovery plan. |
| Supplier Planning | Supplier-facing shortages dominate the recovery workload. Expedites, split shipments, backup sources, and delivery confirmation are required. |
| Production Planning | Build sequencing may need to prioritize available kits while blocked kits are recovered. |
| Quality | Safety-critical shortages such as crash structure components require quality involvement before workaround or substitution. |
| Leadership | The dashboard highlights which systems need attention first: battery pack, thermal system, and drive unit. |

## Risk vs. quantity insight

One of the most important lessons from the case study is that shortage quantity and production risk are related but not identical. Battery Pack System has the largest total missing quantity. However, Electric Drive Unit and Chassis/Body items can be highly critical even with smaller missing quantities. A good operations dashboard therefore needs both quantity views and severity/risk views.

# 9. Recommendations

## 9.1 Prioritize line-stopper shortages before general replenishment

The shortage tracker should be reviewed daily until all line-stopper shortages are closed. The priority sequence should start with Critical severity, then High severity, then Medium. The team should focus on parts that block major assembly stages: Electric Motor Assembly, Front Crash Beam, Resolver / Position Sensor, Gearbox Bearings, Temperature Sensors, Cooling Plates, Pack Coolant Tubes, Copper Busbars, Aluminum Busbars, and Module Insulation Sheets.

## 9.2 Use kit-level release decisions

The plant should not release kits based only on inventory status. It should release kits only after the kitting tracker confirms that picked quantity equals required quantity or that leadership has approved a partial release. Kit-level readiness is more operationally useful than item-level inventory alone.

## 9.3 Create a daily shortage war-room process

Review all 17 shortage IDs at the start of the day.

Confirm whether expected recovery dates are still valid.

Separate supplier-owned shortages from warehouse-owned and production-owned shortages.

Escalate any shortage with a recovery date later than the target kit release date.

Update production planning if any kit remains on hold after the recovery deadline.

## 9.4 Improve supplier resilience for high-risk components

High-risk components should have backup supplier validation, approved alternates, or additional safety stock where practical. This is especially important for battery pack electronics, thermal components, busbars, insulation materials, drive unit sensors, and bearings.

## 9.5 Expand dashboard filters for operational use

If this dashboard were turned into a real plant tool, it should include filters for Major Assembly, Kit ID, Responsible Owner, Severity, Risk Category, and Escalation Status. These filters would allow different teams to use the same dashboard for different review meetings.

# 10. Limitations and Future Improvements

## Limitations

The model uses mock data and is not an official EV OEM bill of materials.

Supplier names and risks are used for educational modeling and are not verified live procurement data.

Risk scores are simplified and weighted manually rather than calculated from live supplier performance history.

Inventory values are modeled for a static 25-vehicle production batch rather than rolling demand over time.

The model does not include transportation lead-time variability, purchase orders, receiving inspection defects, or actual warehouse location scans.

The Tableau dashboard summarizes the model but does not update from live ERP/WMS/MRP transactions.

## Future improvements

Add purchase order status, supplier promise dates, and actual receipt dates.

Add warehouse storage location and line-side delivery route.

Add defect/quality hold status for incoming components.

Add supplier scorecard history and live lead-time performance.

Add a production schedule and model kit release by build day.

Create a Tableau dashboard page for supplier risk and another page for shortage recovery aging.

Add scenario analysis for a battery-cell delay, semiconductor delay, supplier shutdown, or expedited freight option.

Add formulas that automatically join the Risk Scoring Model to Kitting and Shortage data so scores stay aligned when rows change.

# Appendix A. Active Shortage Tracker Summary

The table below summarizes the 17 active shortages used in the Tableau dashboard and Shortage Tracker.

| ID | Component | Major Assembly | Missing | Severity | Owner | Status | Production Impact |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SHRT-001 | Temperature Sensors | Battery Pack System | 140 | High | Buyer / Supplier Planner | Escalated | Battery monitoring installation at risk |
| SHRT-002 | Battery Cooling Plates | Battery Pack System | 35 | High | Buyer / Supplier Planner | Escalated | Battery thermal assembly at risk |
| SHRT-003 | Pack Coolant Tubes | Battery Pack System | 70 | High | Warehouse / Supplier Planner | Escalated | Battery thermal installation at risk |
| SHRT-004 | Battery Pack Gasket Seal | Battery Pack System | 11 | High | Buyer / Supplier Planner | Escalated | Battery pack sealing cannot complete |
| SHRT-005 | Electric Motor Assembly | Electric Drive Unit | 6 | Critical | Production Planner | Escalated | Drive unit assembly at risk |
| SHRT-006 | Onboard Charger | Power Electronics and Charging System | 1 | Medium | Buyer / Supplier Planner | Open | Charging system kit incomplete |
| SHRT-007 | Front Crash Beam | Chassis and Body Structure | 6 | Critical | Quality + Supplier Planner | Escalated | Body crash structure assembly at risk |
| SHRT-008 | Coolant Fluid | Thermal Management System | 120 | High | Warehouse / Maintenance Planner | Open | Thermal system filling delayed |
| SHRT-009 | Springs | Suspension, Brakes, and Wheels | 38 | High | Buyer / Supplier Planner | Escalated | Suspension installation at risk |
| SHRT-010 | Copper Busbars | Battery Pack System | 120 | High | Buyer / Supplier Planner | Escalated | Battery module electrical assembly at risk |
| SHRT-011 | Aluminum Busbars | Battery Pack System | 80 | High | Buyer / Supplier Planner | Escalated | Battery module electrical assembly at risk |
| SHRT-012 | Module Insulation Sheets | Battery Pack System | 70 | High | Buyer / Supplier Planner | Escalated | Battery module safety insulation delayed |
| SHRT-013 | Copper Windings | Electric Drive Unit | 1 | Medium | Buyer / Supplier Planner | Open | Motor winding installation incomplete |
| SHRT-014 | Motor Bearings | Electric Drive Unit | 9 | High | Buyer / Supplier Planner | Escalated | Motor bearing installation at risk |
| SHRT-015 | Resolver / Position Sensor | Electric Drive Unit | 9 | Critical | Buyer / Supplier Planner | Escalated | Motor sensor installation at risk |
| SHRT-016 | Gearbox Bearings | Electric Drive Unit | 65 | Critical | Buyer / Supplier Planner | Escalated | Gearbox assembly at risk |
| SHRT-017 | Gearbox Lubricant | Electric Drive Unit | 3 | Medium | Warehouse / Maintenance Planner | Open | Gearbox final assembly delayed |

# Appendix B. Top Risk Components

The following table shows the highest-scoring components in the Risk Scoring Model. These are the components the dashboard ranks as top production risk items.

| Component | Major Assembly | Supplier | Score | Category | Recommended Mitigation |
| --- | --- | --- | --- | --- | --- |
| Single-Speed Gearbox | Electric Drive Unit | GKN Automotive | 4.70 | Critical | Escalate immediately, expedite supply, and review backup source |
| Inverter Control PCB | Electric Drive Unit | Jabil | 4.70 | Critical | Escalate immediately, expedite supply, and review backup source |
| Electric Motor Assembly | Electric Drive Unit | BorgWarner | 4.70 | Critical | Escalate immediately, expedite supply, and review backup source |
| Thermal Interface Pads | Battery Pack System | 3M | 4.50 | Critical | Escalate immediately, expedite supply, and review backup source |
| Module Insulation Sheets | Battery Pack System | DuPont | 4.50 | Critical | Escalate immediately, expedite supply, and review backup source |
| Stator Core | Electric Drive Unit | Schaeffler | 4.50 | Critical | Escalate immediately, expedite supply, and review backup source |
| Charge Port Assembly | Power Electronics and Charging System | TE Connectivity | 4.45 | Critical | Escalate immediately, expedite supply, and review backup source |
| Current Sensor | Battery Pack System | LEM | 4.30 | Critical | Escalate immediately, expedite supply, and review backup source |
| Rear Control Arms | Suspension, Brakes, and Wheels | ZF Friedrichshafen | 4.20 | High | Create recovery plan and monitor daily |
| Resolver / Position Sensor | Electric Drive Unit | Sensata Technologies | 4.20 | High | Create recovery plan and monitor daily |

## Risk category breakdown

| Risk Category | Component Count |
| --- | --- |
| Critical | 8 |
| High | 15 |
| Medium | 72 |
| Low | 7 |

# Appendix C. Portfolio Positioning

This project can be positioned as a practical supply chain portfolio case study because it shows how a student can turn operational concepts into a structured model and dashboard. The project demonstrates more than spreadsheet usage. It demonstrates an understanding of how BOMs, suppliers, inventory, kitting, shortages, and risk management connect inside complex assembly logistics.

## Suggested portfolio title

EV Assembly Logistics Readiness Dashboard and Case Study

## Suggested portfolio description

Built a multi-sheet EV assembly logistics model connecting BOM structure, supplier sourcing, inventory availability, kitting readiness, shortage tracking, risk scoring, and Tableau dashboarding. The project simulates a 25-vehicle production batch and identifies material shortages, line-stopper risks, responsible owners, recovery actions, and high-risk components across major EV systems.

## Skills demonstrated

BOM structure and component planning

Supplier master data and sourcing risk logic

Inventory availability and shortage calculation

Kitting readiness and line-stopper analysis

Shortage escalation and recovery planning

Risk scoring and mitigation recommendations

Tableau dashboard design for production readiness

Operations storytelling for supply chain decision-making

## Final reflection

The finished case study shows how production logistics depends on much more than having a parts list. It requires connecting material requirements, supplier capability, inventory readiness, warehouse picking, production sequencing, shortage ownership, and management visibility. The model gives a realistic view of how supply chain teams support complex assembly operations.